Example for the th_weighted.exe Program

By Tom Irvine

Email:  tomirvine@aol.com

________________________________________________________________________

Input File

The sample input file is called is seat.txt.  It is a plain text ASCII file with two columns of numbers.

The first column is time in seconds.  The second column is accelerations in m/sec^2.  The acceleration could also be in units of G.  The two columns should have either a space or a tab between them.  The numerical data is “e” format, but it could have been in simple floating format.

The file could contain header lines, but it does not in this case.

The complete seat.txt file contains 300,000 lines.  Here are the first few lines:

0.00000e+000   -1.8556e-003

1.00000e-003    7.2144e-001

2.00000e-003   -2.4430e+000

3.00000e-003    1.3543e+000

4.00000e-003    4.0365e+000

5.00000e-003    6.3102e-001

6.00000e-003    3.1626e+000

7.00000e-003   -1.1772e+000

8.00000e-003   -3.3336e-001

9.00000e-003   -5.4432e-001

1.00000e-002   -7.5966e+000

1.10000e-002   -4.2378e-001

Users who have data in other formats should contact Tom Irvine for help with a conversion utility.

Program Execution

The th_weighted.exe program runs in DOS or console mode.  It can be executed from the DOS prompt or by using the Windows “My Computer” icon.  A screenshot of the input is shown on the next page.  The colors of the text and background have been inverted for clarity.

The program is interactive.  It prompts the user for the input parameters as shown in the image above.

The program then prints some intermediate lines to the screen, which are omitted in this tutorial for brevity.  These are esoteric data processing parameters which the user may wish to ignore.

The program then prompts the user for the time window duration.  A window of 60 seconds is used for this example.  The screenshot is given on the next page.

The screenshot shows the AW weighted output levels for each 60 second interval.

The program also gives the composite AW, as well as the MTW, VDV, and MTW/AW ratio.

The output is also stored in a text file.  The output filename for this example is called: seat_weighted.txt.  It is shown on the next page.

The output filename was specified as an input parameter in the first screenshot, on the previous page.

Input file = seat.txt

Input unit = m/sec^2

Weighting = Wk

Time(sec) 	 AW(m/sec^2)RMS

30 	       0.6179

90 	       0.6046

150 	       0.6168

210 	       0.6013

270 	        0.608

Composite Weighted Level    AW =        0.6098 (m/sec^2)RMS

Maximum Transient Vibration MTW =        0.6179 (m/sec^2)RMS

Fourth Power Vibration Dose VDV =        0.9119 (m/sec^(1.75))

MTW/AW =         1.013